Over5protocol.doc

-----------------

Copyright (c) 1995,1996 Daniel Kahlin <tlr@stacken.kth.se>

(last changed 960402)

A bit incomplete, but should work for now.

---------------------------------------------------------------------------

Over5 low level protocol specification

general overview by Daniel Kahlin <tlr@stacken.kth.se>

The low level end of the protocol is able to send and receive data blocks

1 to 256 bytes in size.  The protocol ensures that the data is correctly

transferred.

T  ||   name: BASIC BLOCK TRANSFER

I  ||   type: low level specification

M  ||   structures: <TRBLK>, <ACKBLK>

E  \/

Sender                                            Receiver

------                                            --------

1a  SEND <TRBLK>_

\__________________________________

\_

1b                                                    RECEIVE <TRBLK>

2a                                                   _RESPOND <ACKBLK>

________________________________/

_/

2b  CHECK <ACKBLK>

T  ||   name: BLOCK TRANSFER WITH RESEND

I  ||   type: low level specification

M  ||   structures: <TRBLK>, <ACKBLK>

E  \/

Sender                                            Receiver

------                                            --------

1a  SEND <TRBLK>_

\__________X___X___________________

\_

1b                                                    RECEIVE <TRBLK>

2a                                                   _RESPOND <ACKBLK>

________________________________/

_/

2b  CHECK <ACKBLK>

3a  SEND <TRBLK>_

\__________________________________

\_

3b                                                    RECEIVE <TRBLK>

4a                                                   _RESPOND <ACKBLK>

________________________________/

_/

4b  CHECK <ACKBLK>

#define HEAD 0x7e

#define CHANNEL_DATA    1

#define CHANNEL_COMMAND 15

#define ST_OK  0x10

#define ST_RESEND   0x20

<TRBLK>

0x00  UBYTE trblk_head;        (=HEAD)

0x01  UBYTE trblk_channel;      xy  y=channel  x=status  (status must be 0

when doing blocksend)

0x02  UBYTE trblk_blocknum;    blocknumber (0-15)

0x03  UBYTE trblk_size;        size of the data block  (0x00 => 256 bytes)

0x04  UBYTE trblk_headsum;     xor of all the bytes up to trblk_headsum and

including trblk_head must be 0

0x05  UBYTE trblk_body[trblk_size];    the data

0x05+trblk_size

UBYTE trblk_bodysum;     xor of the body and trblk_bodysum must be 0

<ACKBLK>

0x00  UBYTE ackblk_head;       (=HEAD)

0x01  UBYTE ackblk_channel;     xy  y=channel  x=status

(ST_OK or ST_RESEND)

0x02  UBYTE ackblk_blocknum;   blocknumber (0-15)

0x03  UBYTE ackblk_size;       size of the data block  (0x00 => 256 bytes)

0x04  UBYTE ackblk_headsum;    xor of all the bytes up to ackblk_headsum and

including ackblk_head must be 0

/END

---------------------------------------------------------------------------

Over5 high level protocol specification

general overview by Daniel Kahlin <tlr@stacken.kth.se>

Contents:

TYPE_MEMTRANSFER:

WRITEMEM

READMEM

SYS

RUN

TYPE_DISKCOMMAND:

DIRECTORY

STATUS

COMMAND

TYPE_FILETRANSFER:

WRITEFILE

READFILE

TYPE_RAWDISKTRANSFER:

WRITETRACK

READTRACK

WRITESECTOR

READSECTOR

TYPE_SERVER:

SERVERLOAD

SERVERSAVE

SERVERCOMMAND

SERVERREADSTRING

*** WRITEMEM *** WRITEMEM *** WRITEMEM *** WRITEMEM *** WRITEMEM *** WRITEM

T  ||   name: WRITEMEM (part of TYPE_MEMTRANSFER)

I  ||   type: high level specification

M  ||   structures: <WMHD>, <RS>

E  \/

Amiga                                             c64

-----                                             ---

1a  SEND <WMHD>_

\___________________________________

\_

1b                                                    RECEIVE <WMHD>

2a                                                   _RESPOND <RS>

________________________________/

_/

2b  CHECK RESPONSE

-- if (RESP!=OK) -> FAIL

3a  SEND BODY_

\_____________________________________

\_

3b                                                    RECEIVE BODY

4a                                                   _RESPOND <RS>

________________________________/

_/

4b  CHECK RESPONSE

*** READMEM *** READMEM *** READMEM *** READMEM *** READMEM *** READMEM ***

T  ||   name: READMEM (part of TYPE_MEMTRANSFER)

I  ||   type: high level specification

M  ||   structures: <RMHD>, <RS>

E  \/

Amiga                                             c64

-----                                             ---

1a  SEND <RMHD>_

\___________________________________

\_

1b                                                    RECEIVE <RMHD>

2a                                                   _RESPOND <RS>

________________________________/

_/

2b  CHECK RESPONSE

-- if (RESP!=OK) -> FAIL

3a                                                   _SEND BODY

__________________________________/

_/

3b  RECEIVE BODY

4a  RESPOND <RS>_

\__________________________________

\_

4b                                                    CHECK RESPONSE

*** SYS *** SYS *** SYS *** SYS *** SYS *** SYS *** SYS *** SYS *** SYS ***

T  ||   name: SYS (part of TYPE_MEMTRANSFER)

I  ||   type: high level specification

M  ||   structures: <SYHD>, <RS>

E  \/

Amiga                                             c64

-----                                             ---

1a  SEND <SYHD>_

\___________________________________

\_

1b                                                    RECEIVE <SYHD>

2a                                                   _RESPOND <RS>

________________________________/

_/

2b  CHECK RESPONSE

*** RUN *** RUN *** RUN *** RUN *** RUN *** RUN *** RUN *** RUN *** RUN ***

T  ||   name: RUN (part of TYPE_MEMTRANSFER)

I  ||   type: high level specification

M  ||   structures: <RUHD>, <RS>

E  \/

Amiga                                             c64

-----                                             ---

1a  SEND <RUHD>_

\___________________________________

\_

1b                                                    RECEIVE <RUHD>

2a                                                   _RESPOND <RS>

________________________________/

_/

2b  CHECK RESPONSE

*** DIRECTORY *** DIRECTORY *** DIRECTORY *** DIRECTORY *** DIRECTORY *** D

T  ||   name: DIRECTORY (part of TYPE_DISKCOMMAND)

I  ||   type: high level specification

M  ||   structures: <DRHD>, <DRRS>, <RS>

E  \/

Amiga                                             c64

-----                                             ---

1a  SEND <DRHD>_

\___________________________________

\_

1b                                                    RECEIVE <DRHD>

2a                                                   _RESPOND <RS>

________________________________/

_/

2b  CHECK RESPONSE

-- if (RESP!=OK) -> FAIL

3a                                                   _RESPOND <DRRS>

________________________________/

_/

3b  CHECK RESPONSE

-- if (RESP!=OK) -> FAIL

4a  RESPOND <RS>_

\__________________________________

\_

4b                                                    CHECK RESPONSE

5a                                                   _SEND BODY

__________________________________/

_/

5b  RECEIVE BODY

6a  RESPOND <RS>_

\__________________________________

\_

6b                                                    CHECK RESPONSE

*** STATUS *** STATUS *** STATUS *** STATUS *** STATUS *** STATUS *** STATU

T  ||   name: STATUS (part of TYPE_DISKCOMMAND)

I  ||   type: high level specification

M  ||   structures: <STHD>, <RS>

E  \/

Amiga                                             c64

-----                                             ---

1a  SEND <STHD>_

\___________________________________

\_

1b                                                    RECEIVE <STHD>

2a                                                   _RESPOND <RS>

________________________________/

_/

2b  CHECK RESPONSE

-- if (RESP!=OK) -> FAIL

3a                                                   _SEND BLOCK

_________________________________/

_/

3b  RECEIVE BLOCK

4a  RESPOND <RS>_

\__________________________________

\_

4b                                                    CHECK RESPONSE

*** COMMAND *** COMMAND *** COMMAND *** COMMAND *** COMMAND *** COMMAND ***

T  ||   name: COMMAND (part of TYPE_DISKCOMMAND)

I  ||   type: high level specification

M  ||   structures: <CMHD>, <RS>

E  \/

Amiga                                             c64

-----                                             ---

1a  SEND <CMHD>_

\___________________________________

\_

1b                                                    RECEIVE <CMHD>

2a                                                   _RESPOND <RS>

________________________________/

_/

2b  CHECK RESPONSE

-- if (RESP!=OK) -> FAIL

3a  SEND BLOCK_

\___________________________________

\_

3b                                                   RECEIVE BLOCK

4a                                                   _RESPOND <RS>

________________________________/

_/

4b  CHECK RESPONSE

*** WRITEFILE *** WRITEFILE *** WRITEFILE *** WRITEFILE *** WRITEFILE *** W

T  ||   name: WRITEFILE (part of TYPE_FILETRANSFER)

I  ||   type: high level specification

M  ||   structures: <WFHD>, <RS>

E  \/

Amiga                                             c64

-----                                             ---

1a  SEND <WFHD>_

\___________________________________

\_

1b                                                    RECEIVE <WFHD>

2a                                                   _RESPOND <RS>

________________________________/

_/

2b  CHECK RESPONSE

-- if (RESP!=OK) -> FAIL

3a  SEND BODY_

\_____________________________________

\_

3b                                                    RECEIVE BODY

4a                                                   _RESPOND <RS>

________________________________/

_/

4b  CHECK RESPONSE

*** READFILE *** READFILE *** READFILE *** READFILE *** READFILE *** READFI

T  ||   name: READFILE (part of TYPE_DISKCOMMAND)

I  ||   type: high level specification

M  ||   structures: <RFHD>, <RFRS>, <RS>

E  \/

Amiga                                             c64

-----                                             ---

1a  SEND <RFHD>_

\___________________________________

\_

1b                                                    RECEIVE <RFHD>

2a                                                   _RESPOND <RS>

________________________________/

_/

2b  CHECK RESPONSE

-- if (RESP!=OK) -> FAIL

3a                                                   _RESPOND <RFRS>

________________________________/

_/

3b  CHECK RESPONSE

-- if (RESP!=OK) -> FAIL

4a  RESPOND <RS>_

\__________________________________

\_

4b                                                    CHECK RESPONSE

5a                                                   _SEND BODY

__________________________________/

_/

5b  RECEIVE BODY

6a  RESPOND <RS>_

\__________________________________

\_

6b                                                    CHECK RESPONSE

*** WRITETRACK *** WRITETRACK *** WRITETRACK *** WRITETRACK *** WRITETRACK

T  ||   name: WRITETRACK (part of TYPE_RAWDISKTRANSFER)

I  ||   type: high level specification

M  ||   structures: <WTHD>, <RS>

E  \/

Amiga                                             c64

-----                                             ---

1a  SEND <WTHD>_

\___________________________________

\_

1b                                                    RECEIVE <WTHD>

2a                                                   _RESPOND <RS>

________________________________/

_/

2b  CHECK RESPONSE

-- if (RESP!=OK) -> FAIL

3a  SEND BODY_

\_____________________________________

\_

3b                                                    RECEIVE BODY

4a                                                   _RESPOND <RS>

________________________________/

_/

4b  CHECK RESPONSE

*** READTRACK *** READTRACK *** READTRACK *** READTRACK *** READTRACK *** R

T  ||   name: READTRACK (part of TYPE_RAWDISKTRANSFER)

I  ||   type: high level specification

M  ||   structures: <RTHD>, <RS>

E  \/

Amiga                                             c64

-----                                             ---

1a  SEND <RTHD>_

\___________________________________

\_

1b                                                    RECEIVE <RTHD>

2a                                                   _RESPOND <RS>

________________________________/

_/

2b  CHECK RESPONSE

-- if (RESP!=OK) -> FAIL

3a                                                   _RESPOND <RS>

________________________________/

_/

3b  CHECK RESPONSE

-- if (RESP!=OK) -> FAIL

4a  RESPOND <RS>_

\__________________________________

\_

4b                                                    CHECK RESPONSE

5a                                                   _SEND BODY

__________________________________/

_/

5b  RECEIVE BODY

6a  RESPOND <RS>_

\__________________________________

\_

6b                                                    CHECK RESPONSE

*** WRITESECTOR *** WRITESECTOR *** WRITESECTOR *** WRITESECTOR *** WRITESE

T  ||   name: WRITESECTOR (part of TYPE_RAWDISKTRANSFER)

I  ||   type: high level specification

M  ||   structures: <WSHD>, <RS>

E  \/

Amiga                                             c64

-----                                             ---

1a  SEND <WSHD>_

\___________________________________

\_

1b                                                    RECEIVE <WSHD>

2a                                                   _RESPOND <RS>

________________________________/

_/

2b  CHECK RESPONSE

-- if (RESP!=OK) -> FAIL

3a  SEND BODY_

\_____________________________________

\_

3b                                                    RECEIVE BODY

4a                                                   _RESPOND <RS>

________________________________/

_/

4b  CHECK RESPONSE

*** READSECTOR *** READSECTOR *** READSECTOR *** READSECTOR *** READSECTOR

T  ||   name: READSECTOR (part of TYPE_RAWDISKTRANSFER)

I  ||   type: high level specification

M  ||   structures: <RSHD>, <RS>

E  \/

Amiga                                             c64

-----                                             ---

1a  SEND <RSHD>_

\___________________________________

\_

1b                                                    RECEIVE <RSHD>

2a                                                   _RESPOND <RS>

________________________________/

_/

2b  CHECK RESPONSE

-- if (RESP!=OK) -> FAIL

3a                                                   _RESPOND <RS>

________________________________/

_/

3b  CHECK RESPONSE

-- if (RESP!=OK) -> FAIL

4a                                                   _SEND BODY

__________________________________/

_/

4b  RECEIVE BODY

5a  RESPOND <RS>_

\__________________________________

\_

5b                                                    CHECK RESPONSE

*** SERVERLOAD *** SERVERLOAD *** SERVERLOAD *** SERVERLOAD *** SERVERLOAD

T  ||   name: SERVERLOAD (part of TYPE_SERVER)

I  ||   type: high level specification

M  ||   structures: <SRVLDHD>, <SRVLDRS>, <SRVLDRS2>, <RS>

E  \/

Amiga                                             c64

-----                                             ---

1a                                                   _SEND <SRVLDHD>

_____________________________/

_/

1b  RECEIVE <SRVLDHD>

2a  RESPOND <SRVLDRS>_

\_____________________________

\_

2b                                                    CHECK RESPONSE

--                                                   if (RESP!=OK) -> FAIL

3a                                                   _RESPOND <SRVLDRS2>

________________________________/

_/

3b  CHECK RESPONSE

-- if (RESP!=OK) -> FAIL

4a  SEND BODY_

\_____________________________________

\_

4b                                                    RECEIVE BODY

5a                                                   _RESPOND <RS>

________________________________/

_/

5b  CHECK RESPONSE

*** SERVERSAVE *** SERVERSAVE *** SERVERSAVE *** SERVERSAVE *** SERVERSAVE

T  ||   name: SERVERSAVE (part of TYPE_SERVER)

I  ||   type: high level specification

M  ||   structures: <SRVSVHD>, <RS>

E  \/

Amiga                                             c64

-----                                             ---

1a                                                   _SEND <SRVSVHD>

_____________________________/

_/

1b  RECEIVE <SRVSVHD>

2a  RESPOND <RS>_

\__________________________________

\_

2b                                                    CHECK RESPONSE

--                                                   if (RESP!=OK) -> FAIL

3a                                                   _SEND BODY

__________________________________/

_/

3b  RECEIVE BODY

4a  RESPOND <RS>_

\__________________________________

\_

4b                                                    CHECK RESPONSE

*** SERVERCOMMAND *** SERVERCOMMAND *** SERVERCOMMAND *** SERVERCOMMAND ***

T  ||   name: SERVERCOMMAND (part of TYPE_SERVER)

I  ||   type: high level specification

M  ||   structures: <SRVCMHD>, <RS>

E  \/

Amiga                                             c64

-----                                             ---

1a                                                   _SEND <SRVCMHD>

_____________________________/

_/

1b  RECEIVE <SRVCMHD>

2a  RESPOND <RS>_

\__________________________________

\_

2b                                                    CHECK RESPONSE

--                                                   if (RESP!=OK) -> FAIL

3a  RESPOND <RS>_

\__________________________________

\_

3b                                                    CHECK RESPONSE

*** SERVERREADSTRING *** SERVERREADSTRING *** SERVERREADSTRING *** SERVERRE

T  ||   name: SERVERREADSTRING (part of TYPE_SERVER)

I  ||   type: high level specification

M  ||   structures: <SRVRSHD>, <SRVRSRS>, <RS>

E  \/

Amiga                                             c64

-----                                             ---

1a                                                   _SEND <SRVRSHD>

_____________________________/

_/

1b  RECEIVE <SRVRSHD>

2a  RESPOND <SRVRSRS>_

\_____________________________

\_

2b                                                    CHECK RESPONSE

--                                                   if (RESP!=OK) -> FAIL

3a                                                   _RESPOND <RS>

________________________________/

_/

3b  CHECK RESPONSE

4a  SEND BODY_

\_____________________________________

\_

4b                                                    RECEIVE BODY

/END

---------------------------------------------------------------------------

Over5 high level protocol specification

structures in detail by Daniel Kahlin <tlr@stacken.kth.se>

All information sent on CHANNEL_COMMAND should begin with the structures

<OHD> and <RS>.  This allows us to easily detect protocol errors.

<OHD>                    general request

0x00  UBYTE ohd_type;

0x01  UBYTE ohd_subtype;

<RS>                     general response

0x00  UBYTE rs_response;

The distinction between an <OHD> and an <RS> can be made using the first

byte.  If MSB is 1, it is an <RS> else it is an <OHD>.

The list below contains all types and responses.

#define TYPE_MEMTRANSFER     0x01

#define TYPE_FILETRANSFER    0x02

#define TYPE_DISKCOMMAND     0x03

#define TYPE_TESTCOMMAND     0x04

#define TYPE_SERVER          0x05

#define TYPE_RAWDISKTRANSFER 0x06

#define RESP_OK              0x80

#define RESP_NOTSUPPORTED    0x81

#define RESP_ERROR           0x82

#define RESP_STRING          0x83

The 'type' field can be used to easily determine if a certain chunk of

commands are supported.  When the 'type' field has been evaluated you

should jump to appropriate routine which in its turn evaluates the

'subtype' field. These subtypes apply:

#define SUB_MT_WRITEMEM      0x01     ;TYPE_MEMTRANSFER

#define SUB_MT_READMEM       0x02

#define SUB_MT_SYS           0x03

#define SUB_MT_RUN           0x04

#define SUB_FT_WRITEFILE     0x01     ;TYPE_FILETRANSFER

#define SUB_FT_READFILE      0x02

#define SUB_DC_DIRECTORY     0x01     ;TYPE_DISKCOMMAND

#define SUB_DC_STATUS        0x02

#define SUB_DC_COMMAND       0x03

#define SUB_TC_BLOCKTEST     0x01     ;TYPE_TESTCOMMAND

#define SUB_TC_FILETEST      0x02

#define SUB_SRV_LOAD         0x01     ;TYPE_SERVER

#define SUB_SRV_SAVE         0x02

#define SUB_SRV_COMMAND      0x03

#define SUB_SRV_READSTRING   0x04

#define SUB_RT_WRITETRACK    0x01     ;TYPE_RAWDISKTRANSFER

#define SUB_RT_READTRACK     0x02

#define SUB_RT_WRITESECTOR   0x03

#define SUB_RT_READSECTOR    0x04

Here follows a detailed specification of all structures currently

used by the high level protocol.

<WMHD>

0x00  UBYTE wmhd_type;        (=TYPE_MEMTRANSFER)

0x01  UBYTE wmhd_subtype;     (=SUB_MT_WRITEMEM)

0x02  UBYTE wmhd_start_l; \ start address in c64 memory

0x03  UBYTE wmhd_start_h; /

0x04  UBYTE wmhd_end_l;  \ end address in c64 memory

0x05  UBYTE wmhd_end_h;  /

<RMHD>

0x00  UBYTE rmhd_type;        (=TYPE_MEMTRANSFER)

0x01  UBYTE rmhd_subtype;     (=SUB_MT_READMEM)

0x02  UBYTE rmhd_start_l; \ start address in c64 memory

0x03  UBYTE rmhd_start_h; /

0x04  UBYTE rmhd_end_l;  \ end address in c64 memory

0x05  UBYTE rmhd_end_h;  /

<SYHD>

0x00  UBYTE syhd_type;        (=TYPE_MEMTRANSFER)

0x01  UBYTE syhd_subtype;     (=SUB_MT_SYS)

0x02  UBYTE syhd_pc_l; \   6502 program counter

0x03  UBYTE syhd_pc_h; /

0x04  UBYTE syhd_memory;   $01 value for the c64

0x05  UBYTE syhd_sr;       6502 status register

0x06  UBYTE syhd_ac;       6502 accumulator

0x07  UBYTE syhd_xr;       6502 X register

0x08  UBYTE syhd_yr;       6502 Y register

0x09  UBYTE syhd_sp;       6502 stack pointer

<RUHD>

0x00  UBYTE ruhd_type;        (=TYPE_MEMTRANSFER)

0x01  UBYTE ruhd_subtype;     (=SUB_MT_RUN)

0x02  UBYTE ruhd_lowmem_l; \   start of basic program

0x03  UBYTE ruhd_lowmem_h; /

0x04  UBYTE ruhd_himem_l; \   end of basic program

0x05  UBYTE ruhd_himem_h; /

<DRHD>

0x00  UBYTE drhd_type;        (=TYPE_DISKCOMMAND)

0x01  UBYTE drhd_subtype;     (=SUB_DC_DIRECTORY)

0x02  UBYTE drhd_pad[2];

0x04  UBYTE drhd_device;        device number

0x05  UBYTE drhd_filename[24];  filename

<DRRS>

0x00  UBYTE drrs_response;    response code

0x01  UBYTE drrs_len_l; \      length of directory in bytes

0x02  UBYTE drrs_len_h; /

<STHD>

0x00  UBYTE sthd_type;        (=TYPE_DISKCOMMAND)

0x01  UBYTE sthd_subtype;     (=SUB_DC_STATUS)

0x02  UBYTE sthd_pad[2];

0x04  UBYTE sthd_device;        device number

<CMHD>

0x00  UBYTE cmhd_type;        (=TYPE_DISKCOMMAND)

0x01  UBYTE cmhd_subtype;     (=SUB_DC_COMMAND)

0x02  UBYTE cmhd_pad[2];

0x04  UBYTE cmhd_device;        device number

<WFHD>

0x00  UBYTE wfhd_type;         (=TYPE_FILETRANSFER)

0x01  UBYTE wfhd_subtype;      (=SUB_FT_WRITEFILE)

0x02  UBYTE wfhd_len_l;    \   length of file

0x03  UBYTE wfhd_len_h;    /

0x04  UBYTE wfhd_device;        device number

0x05  UBYTE wfhd_filename[24];  filename

<RFHD>

0x00  UBYTE rfhd_type;         (=TYPE_FILETRANSFER)

0x01  UBYTE rfhd_subtype;      (=SUB_FT_READFILE)

0x02  UBYTE rfhd_pad[2];

0x04  UBYTE rfhd_device;        device number

0x05  UBYTE rfhd_filename[24];  filename

<RFRS>

0x00  UBYTE rfrs_response;    response code

0x01  UBYTE rfrs_len_l; \      length of file in bytes

0x02  UBYTE rfrs_len_h; /

<WTHD>

0x00  UBYTE wthd_type;         (=TYPE_RAWDISKTRANSFER)

0x01  UBYTE wthd_subtype;      (=SUB_RT_WRITETRACK)

0x02  UBYTE wthd_track;        track to write

0x03  UBYTE wthd_numtracks;    number of tracks to be written

0x04  UBYTE wthd_device;        device number

<RTHD>

0x00  UBYTE wthd_type;         (=TYPE_RAWDISKTRANSFER)

0x01  UBYTE wthd_subtype;      (=SUB_RT_READTRACK)

0x02  UBYTE wthd_track;        track to read

0x03  UBYTE wthd_numtracks;    number of tracks to be read

0x04  UBYTE wthd_device;        device number

<RTRS>

0x00  UBYTE rtrs_response;    response code

0x01  UBYTE rtrs_sectors;      number of sectors

<WSHD>

0x00  UBYTE wshd_type;         (=TYPE_RAWDISKTRANSFER)

0x01  UBYTE wshd_subtype;      (=SUB_RT_WRITESECTOR)

0x02  UBYTE wshd_track;        track to write

0x03  UBYTE wshd_sector;       sector to write

0x04  UBYTE wshd_device;        device number

<RSHD>

0x00  UBYTE rshd_type;         (=TYPE_RAWDISKTRANSFER)

0x01  UBYTE rshd_subtype;      (=SUB_RT_READSECTOR)

0x02  UBYTE rshd_track;        track to read

0x03  UBYTE rshd_sector;       sector to read

0x04  UBYTE rshd_device;        device number

<BTHD>

0x00  UBYTE bthd_type;         (=TYPE_TESTCOMMAND)

0x01  UBYTE bthd_subtype;      (=SUB_TC_BLOCKTEST)

<FTHD>

0x00  UBYTE fthd_type;         (=TYPE_TESTCOMMAND)

0x01  UBYTE fthd_subtype;      (=SUB_TC_FILETEST)

0x02  UBYTE fthd_len_l;  \  length of test data in bytes

0x03  UBYTE fthd_len_h;  /

<FTRS>

0x00  UBYTE ftrs_response;    response code

0x01  UBYTE ftrs_len_l; \      length of file in bytes

0x02  UBYTE ftrs_len_h; /

<SRVLDHD>

0x00  UBYTE srvldhd_type;         (=TYPE_SERVER)

0x01  UBYTE srvldhd_subtype;      (=SUB_SRV_LOAD)

0x02  UBYTE srvldhd_pad[4];

0x06  UBYTE srvldhd_filename[64];  filename to load

<SRVLDRS>

0x00  UBYTE srvldrs_response;    response code

0x01  UBYTE srvldrs_start_l; \      start address of file

0x02  UBYTE srvldrs_start_h; /

0x03  UBYTE srvldrs_end_h; \     end address of file

0x04  UBYTE srvldrs_end_h; /

<SRVLDRS2>

0x00  UBYTE srvldrs2_response;    response code

0x01  UBYTE srvldrs2_start_l; \      new start address

0x02  UBYTE srvldrs2_start_h; /

<SRVSVHD>

0x00  UBYTE srvsvhd_type;         (=TYPE_SERVER)

0x01  UBYTE srvsvhd_subtype;      (=SUB_SRV_SAVE)

0x02  UBYTE srvsvhd_start_l; \      start address of file

0x03  UBYTE srvsvhd_start_h; /

0x04  UBYTE srvsvhd_end_h; \     end address of file

0x05  UBYTE srvsvhd_end_h; /

0x06  UBYTE srvsvhd_filename[64];  filename to load

<SRVCMHD>

0x00  UBYTE srvcmhd_type;         (=TYPE_SERVER)

0x01  UBYTE srvcmhd_subtype;      (=SUB_SRV_COMMAND)

0x02  UBYTE srvcmhd_pad[4];

0x06  UBYTE srvcmhd_command[64];  the command

<SRVRSHD>

0x00  UBYTE srvrshd_type;         (=TYPE_SERVER)

0x01  UBYTE srvrshd_subtype;      (=SUB_SRV_READSTRING)

0x02  UBYTE srvrshd_width;     width of destination screen

0x03  UBYTE srvrshd_height;    height of destination screen

<SRVRSRS>

0x00  UBYTE srvrsrs_response;    response code

0x01  UBYTE srvrsrs_rows;        number of actual rows

/END